ASSEMBLY INSTRUCTIONS

Nava Chandelier (Item#FB150-1170)

Caution:

- FOR YOUR SAFETY: Turn off the power to main electric circuit/ fuse box before beginning work.

Important Safety Instructions:

- Professional installation is recommended for hardwired lighting. Incorrect installation can lead to fires.

- These instructions are provided for your safety. It is very important that they are read carefully and completely before assembling fixture.

- We strongly recommend that two people install the lighting fixture.

- This fixture has been rated for up to (3) 40-watt maximum TYPE B bulbs (not included) or (3) 9-watt compact fluorescent light bulbs (not included). To avoid the risk of fire, do not exceed the recommended wattage.

- This lighting fixture is intended for indoor, residential use.

# Warning:

- To reduce the risk of fire, electrical shock or personal injury, do not turn on lighting fixture until light bulb is installed and assembly is complete.

- Always turn off the lighting fixture and allow it to cool before replacing the light bulb.

# Care instructions:

- Do not polish or use harsh cleaners as they may damage the finish.

- Wipe clean with a non-colored, lint-free soft, dry cloth.

# Pre-assembly:

- Remove all parts and hardware from box along with any plastic protective packaging.

- Check that all parts listed are included.

- Do not discard any contents until after assembly is complete to avoid accidentally discarding small parts or hardware.

# Component List

| # Part code | # Component | # Description | # Qty |
| --- | --- | --- | --- |
| # A |  | # Ceiling cap | # 1 |
| # B |  | # Crossbar | # 1 |
| # C |  | # Hang ring | # 1 |
| # D |  | # Hang chain | # 1 |
| # E |  | # Canopy nut | # 1 |
| # F |  | # Bulb | # Not included |
| # G |  | Quick link chain connector | # 2 |

# Hardware List

| # Part code | # Component | # Description | # Qty |
| --- | --- | --- | --- |
| # I |  | # Mounting screw | # 2 |
| # J |  | # Plastic wire nut connector | # 3 |
| # K |  | # Ground screw | # 1 |
| # L |  | # Hex nut | # 2 |

# To Adjust the Suspension Length of the Fixture:

- The adjustment or shortening of the lighting fixture’s suspension length is only needed if the factory set length is not appropriate for your installation. We recommend that you adjust the lighting fixture to the proper desired length before cutting the electrical cord.

- Determine the lighting fixture suspension length from the ceiling for the product installation, allowing the extra cord length for the outlet junction box wire connections. If needed, cut off the extra power cord length on the 3 wire cords so that they all at least 6 inches (152.4mm) long within the outlet junction box.

# Assembly Instructions:

- Step 1: Slide the Canopy (A) and Canopy nut (E), let them on the Hang chain (D) position.

Step 2: With the help of another person to hold the weight of lighting fixture, attach the Crossbar (B) onto the Junction box by using 2 Mounting screws(I) with Screwdriver, and to make the wire connection.

Adjust the position of the threaded tube of Hang ring (C) and fix the 2 upper Hex nuts(L) tightly to the Crossbar (B). Connect Hand ring (C) with Hang chain (D) by Quick link chain connector (G).

Step 3: Slide the Canopy (A) and Canopy nut (E) upwards. And make sure hold the Canopy(A) in place with the Canopy nut (E).

Step 4: Connect light body（H）with Quick link chain connector（G）.

.

..

Thank you for your purchase