ASSEMBLY INSTRUCTIONS

Conway Set of 2 Wall Sconces (Item#FB155-1173)

Caution:

- FOR YOUR SAFETY: Turn off the power to main electric circuit/ fuse box before beginning work.

Important Safety Instructions:

- Professional installation is recommended for hardwired lighting. Incorrect installation can lead to fires.

- These instructions are provided for your safety. It is very important that they are read carefully and completely before assembling fixture.

- We strongly recommend that two people install the lighting fixture.

- This fixture has been rated for up to (1) 40-watt maximum TYPE B bulbs (not included) or (1) 9-watt compact fluorescent light bulbs (not included). To avoid the risk of fire, do not exceed the recommended wattage.

- This lighting fixture is intended for indoor, residential use.

# Warning:

- To reduce the risk of fire, electrical shock or personal injury, do not turn on lighting fixture until light bulb is installed and assembly is complete.

- Always turn off the lighting fixture and allow it to cool before replacing the light bulb.

# Care instructions:

- Do not polish or use harsh cleaners as they may damage the finish.

- Wipe clean with a non-colored, lint-free soft, dry cloth.

# Page 1 of 3

# Pre-assembly:

- Remove all parts and hardware from box along with any plastic protective packaging.

- Check that all parts listed are included.

- Do not discard any contents until after assembly is complete to avoid accidentally discarding small parts or hardware.

# Component List

| # Part code | # Component | # Description | # Qty |
| --- | --- | --- | --- |
| # A |  | # Ceiling cap | # 1 |
| # B |  | # Crossbar | # 1 |
| # C |  | # Shade | # 1 |
| # D |  | # Socket lock ring | # 1 |
| # E |  | # Bulb | # Not included |

# Hardware List

| # Part code | # Component | # Description | # Qty |
| --- | --- | --- | --- |
| # F |  | # Mounting screw G5/32-T32 L15 | # 2 |
| # G |  | # Mounting screw # G8/32-T32L32 | # 2 |
| # H |  | # Ground screw | # 1 |

# Page 2 of 3

# Assembly Instructions:

Step1：Attach the crossbar (B) onto the electrical junction box by using 2 mounting screws(G).

Step 2: Connect the ceiling cap (A) with crossbar (B), tightly lock by mounting screw (F).

Step 3: Position the lamp shade (C) downwards and tighten with the socket lock ring (D), lastly install the bulb (E not included).

Thank you for your purchase

# Page 3 of 3