# Olliix Tablet Shopping Scanner Pairing

## 2 button to scanner windows 8

- Start barcode scanner. Press trigger for 8sec to [Enter Pairing mode] or scan setup barcodes, blue LED flashing.

- Open Control panel->”Device and Printer”,  and click “Add Device” button,  find [General Bluetooth HID Barcode] device and double click to install it.

- You will hear a beep from scanner if pairing success. You will find it in the paired device.

The Blue LED will be on all the time without flashing.

Pairing mode

Power off barcode:

## 3 Button (Koolertron) Scanner  Windows 8

- From the right edge swipe left to show the system menu

- Click on Settings

- At the bottom of the Settings click on Change PC settings

- Towards the top of the PC Settings menu click on PCs and Devices

- Turn on scanner.  A blinking red light indicates it is ready to pair

- On the PCs and Devices menu click on Devices

- On the Add Devices screen click on Add a device

- The scanner will search for a device.  When found, the scanner device name should start with a CT

- Click on the scanner device

- Click on the Pair button

- A screen should open with a text box for a pairing code.  Enter 10010

- Two beeps should sound indicate the device has been paired

Note:  If the device is not found in step 8 go back to the PC and Devices menu and click on Bluetooth then continue with step 9 above.